Section A:

- Metals can become corroded.

Explain what the term ‘corrosion’ means.

___________________________________________________________________________

___________________________________________________________________________

- Match each key word with the correct definition.

| Electroplating |  | Zinc is used as a sacrificial metal to prevent the corrosion of iron. |
| --- | --- | --- |
| Sacrificial protection |  | Adding a thin layer of metal to an object using electrolysis. |
| Greasing |  | The corrosion of iron. |
| Rusting |  | Adding a slippery substance to a metal to create a barrier to prevent contact with water and oxygen. |
| Galvanise |  | When a metal contains a coating of a more reactive metal so that it is protected from corrosion. |

- State the two substances that must be present for iron to rust.

- ___________________________    2. ___________________________

- An iron gate was painted to prevent rusting.

Explain how painting can be used to prevent the iron gate from rusting.

___________________________________________________________________________

___________________________________________________________________________

- Magnesium is attached on the outside of a steel submarine because…

Tick ( ) one box.

| - it reacts with the steel to prevent corrosion. |  |
| --- | --- |
| - it is more reactive than steel, so prevents corrosion. |  |
| - it is a form of electroplating to prevent corrosion. |  |

- A coating of copper will not sacrificially protect iron from corrosion.

Explain why using your knowledge of the reactivity series.

___________________________________________________________________________

___________________________________________________________________________

Section B

- Electroplating is a technique for preventing corrosion.

Describe how to electroplate a steel spoon with silver.

Draw a labelled diagram.

___________________________________________________________________________

___________________________________________________________________________

___________________________________________________________________________

___________________________________________________________________________

- Some airplane parts are made from aluminium and are not coated

Suggest a reason why they do not corrode.

___________________________________________________________________________

___________________________________________________________________________

___________________________________________________________________________

- Grease is often added to metal bike chains to prevent corrosion.

Explain how greasing prevents corrosion of a bike chain.

___________________________________________________________________________

___________________________________________________________________________

- Explain why copper does not easily corrode.

___________________________________________________________________________

___________________________________________________________________________

- State a similarity and a difference between galvanising and sacrificial protection techniques for preventing corrosion.

Similarity___________________________________________________________________

___________________________________________________________________________

Difference_________________________________________________________________

___________________________________________________________________________

Section C

A student used the following method to investigate the best technique for preventing rusting of iron.

- Measure the mass of a piece of iron using a balance

- Place the untreated iron into an open test tube containing water

- Leave for 1 week

- Measure the final mass of the iron

- Repeat steps 1 to 4 with:

  - iron covered in grease

  - galvanised iron

  - painted iron.

The results are shown below.

| Condition | Starting Mass (g) | Final Mass (g) | Change in mass (g) | Percentage change (%) |
| --- | --- | --- | --- | --- |
| Untreated | 2.74 | 2.98 | + 0.24 | 8.8 |
| Grease | 2.91 | 3.01 |  |  |
| Galvanised | 2.80 | 2.82 |  |  |
| Painted | 2.67 | 2.78 |  |  |

- State a control variable for this experiment.

___________________________________________________________________________

- State the resolution of the balance that was used.  ______________________ g

- Complete the gaps in the results table by:

- calculating the change of mass for each condition

- calculating percentage change of mass for each condition.

- Explain why measuring the change in mass allows the student to compare the extent of rusting.

___________________________________________________________________________

___________________________________________________________________________

- Write a conclusion for these results.

___________________________________________________________________________

___________________________________________________________________________

___________________________________________________________________________

___________________________________________________________________________